# Low noise amplifier parts guide

Revision 4

An LNA can be constructed according to the circuit diagram using small, surface-mount circuit elements and a pre-fabricated printed circuit board.

The pre-fabricated printed circuit board designed for this LNA can be purchased from Oshpark at https://oshpark.com/shared_projects/zvqWjxWi

Most of the circuit parts can be ordered from Digikey at digikey.com, as listed in the table below. The RF shield can be ordered from Leader Tech at leadertechinc.com.

The amplifier chips, U1 – U5, can be obtained from Mini-Circuits at minicircuits.com. You can request free “samples” from the web page.

Parts are grouped by type. Original component values and part numbers are retained.

# Circuit parts

| Circuit Diagram Part | Description | Digikey Part # | Quantity |
| --- | --- | --- | --- |
| C1, C5, C6 | 27 pF | 712-1334-2-ND | 3 |
| C2, C3, C7, C8, C9, C10, C11, C12, C13, C14 | 330 pF | 478-1181-2-ND | 10 |
| C4 | 4700 pF | 399-14206-2-ND | 1 |
| C15 | 6.8 pF | 712-1364-2-ND | 1 |
| C16 | 10 µF | 399-8153-1-ND | 1 |
| R1 | 33 kΩ | 311-33.0KHRTR-ND | 1 |
| R2 | 27 Ω | P26.1HTR-ND | 1 |
| R3, R8 | 50 Ω | RR08Q49.9DTR-ND | 2 |
| R4, R13 | 43 Ω | 311-43.0HRTR-ND | 2 |
| R5 | 4.7 kΩ | 541-4.70KHTR-ND | 1 |
| R6 | 17.4 Ω | 541-17.4HTR-ND | 1 |
| R7, R14 | 5.8 Ω | 311-5.76HRTR-ND | 2 |
| R9, R10 | 287 Ω | 541-287HTR-ND | 2 |
| R11, R12, R15, R16 | 870 Ω | 311-866HRTR-ND | 4 |
| L1, L3, L4, L5, L6 | 100 nH | 490-5664-2-ND | 5 |
| L2 | 10 nH | 490-5648-2-ND | 1 |
| IC1 | LM2940 voltage regulator | LM2937IMPX-5.0/NOPBTR-ND | 1 |
| X1 | CONN SMA RCPT STR 50 OHM PCB | WM5543-ND | 1 |
| X2 | CONN SMA JACK R/A 50 OHM PCB | 931-1361-ND | 1 |

# Other manufacturers

| Circuit Diagram Part | Description | Manufacturer | Part No. |
| --- | --- | --- | --- |
| U1, U3 | Amplifier | Mini-Circuits | GALI 39+ |
| U2 | Amplifier | Mini-Circuits | SAV-541+ |
| U4, U5 | LTCC Band Pass Filter | Mini-Circuits | BFCN-1445+ |
| Circuit Board Shielding | RF shield for circuit board | Leader Tech | 14R-CBSA-0.5X2.75X0.13 |