# DSPIRA 2018 LNA assembly

Pranav Sanghavi | June 22, 2018

Institute reference. Match these component values to your board revision.

Refer to the schematic and layout in Figures 1 and 2 while assembling the LNA.

# Assembly

Assemble all surface mount passives first: resistors, capacitors, and inductors. Work from left to right on the board. Install components with the same value together.

| Component Label | Value |
| --- | --- |
| L2 | 10 nH |
| L1, L3, L4, L5, L6 | 100 nH |
| C6 | 9.1 pF |
| C1, C5 | 27 pF |
| C15 | 6.8 pF |
| C4 | 4700 pF |
| C2, C3, C7, C8, C9, C10, C11, C12, C13, C14 | 330 pF |
| R5 | 4.7k Ω |
| R2 | 27k Ω |
| R1 | 33k Ω |
| R3 | 50 Ω |
| R8 | 150 Ω |
| R4, R13 | 43 Ω |
| R6 | 17.4 Ω |
| R7, R14 | 5.8 Ω |
| R9, R10 | 287 Ω |
| R11, R12, R15, R16 | 870 Ω |

## Now place the actives:

| Component label | Value | Notes |
| --- | --- | --- |
| U2 | SAV-541 | Make sure the large pad lines up with the big foot |
| U1, U3 | GALI-39+ |  |
| U4, U5 | BFCN-1445 | Align dot to point towards input |
| IC1 | LM2940 |  |

# Figure 1: LNA circuit schematic

# Figure 2: LNA board layout