# Multiple signal sources

Student activity

Name ______________________________    Date __________________

In this activity you will construct a program that is a signal generator comprised of multiple sine waveforms with  the ability to control their amplitudes and frequencies. These are added together to create a more complex output.

# Build a Two Waveform Signal Generator

Open your signal source frequency file from Lesson 1.

Save as . . .  using a new file name.

We will be working with sine waveforms.

In the Signal Source block, change the Waveform type to "Sine". Also change the name of Frequency to  "frequency1" and Amplitude to "amplitude1".

In the QT GUI Range block for "frequency", change the ID to "frequency1", and in the QT GUI Range  block for "amplitude", change the ID to "amplitude1".

Add a second signal source by completing the following:

Copy the frequency1 QT GUI Range block, and paste it on the canvas. Then change the ID to  “frequency2”.

Copy the amplitude1 QT GUI Range block, and paste it on the canvas. Open this block, and change ID  to "amplitude2".

Click on the Signal Source block and copy it (right-click Copy, or ctrl-c) and paste it on the canvas.

[Alternately, import a Signal Source block from the library on the right. Be sure to change the Waveform  type to "Sine".]

In this new Signal Source block change the Frequency to "frequency2" and Amplitude to "amplitude2".

Delete the connections from the original Signal Source block. (Highlight and hit "Delete".)

Add an Add block to the canvas. Make sure its IO Type is set to "float".

Connect both Signal Source blocks to the inputs of the Add block, one to each input.

Connect the output of the Add block to the Audio Sink and QT GUI Sink.

Your canvas should look similar to the following:

Run the program.

Explore the graphical displays and audio output while changing the frequencies and amplitudes.

# Build a Three Waveform Signal Generator

Add a third signal source:

Repeat the steps above to add a third Signal Source. Be sure to add the appropriate frequency and  amplitude variables by adding additional QT GUI Range blocks.

Change the Num Inputs in the Add block to "3".

Run the program and explore the output audio and the displays by the changing the frequencies and  amplitudes of each input.

# Build a six Waveform Signal Generator

Add 3 more signal sources to make a signal generator comprised of 6 sine waveforms.

We will be using this in Lessons 3 and 4.