# Horn telescope discussion guide

Teacher reference

a. What is the telescope detecting?

Radio waves

b. What is the source of these radio waves? How do we know?

The signals come from outside the earth

→ If we point the telescope at different likely sources on earth, e.g. buildings, radio

towers, other man-made structures, the “peaks” we observed do not show up. Also, the peak signals are strongest from “empty” sky, and the signal does not vary  significantly over a large range of sky.

The signals are radio waves

→ The telescope consists of an antenna (the wire in the can) and a low-noise amplifier

(LNA) designed to magnify and filter signals for the specific frequency 1420  MHz, which is in the radio frequency range.

c. Horn telescope diagram

i. Hardware

horn & tin can

metal wire

cables

amplifier

dongle

computer

ii. Software

GNU Radio – open source software defined radio program